|  | Image Processing Practical #1 |  |  |
| --- | --- | --- | --- |
| Name | Harshal Kadam | Roll Number | 21306A1024 |
| Subject/Cours e: | Image Processing |  |  |
| Topic | Image Processing Basics |  |  |
| Pixel |  |  |  |
| Program to calculate number of samples required for an image figure; m=4; n=6; N=400; Fs=m*N*n*N; disp(Fs,'Number of samples required to preserve the information in the image='); |  |  |  |
| Image Properties |  |  |  |
| Program to access image properties – Dimension, height, width, number of channels, accessing and modifying any pixel |  |  |  |

Sampling

Program to study the effects of reducing the spatial resolution of a digital image

Quantization

Program to study the effects of varying the number of intensity levels in a digital image i=imread("("C:\Users\student\Downloads \IPCV\images\.png"); subplot(2,2,1); imshow(i); title('original image') i=double(i); k1=(i*255)/64; subplot(2,2,2); k1=uint8(k1); imshow(uint8(k1));

title('Quantization 64'); k2=(i*255)/32; subplot(2,2,3); k2=uint8(k2); imshow(uint8(k2)); title('Quantization 32'); k3=(i*255)/16; subplot(2,2,4); k3=uint8(k3); imshow(uint8(k3));

Image Addition

Program to perform image addition for noise reduction. figure; i=imread("("C:\Users\student\Downloads \IPCV\images\.png"); i=imnoise(i,'salt & pepper',0.001); subplot(1,2,1); imshow(i); title('noisy Image'); k=imadd(i,50); //I +30 subplot(1,2,2); imshow(k); title('Smooth Image after performing image addition');

Image Subtraction

Program to compare images using subtraction for enhancing the difference between images figure; i=imread('C:\Program Files\scilab-6.0.2\IPCV\images\tool1.png'); j=imread('C:\Program Files\scilab-6.0.2\IPCV\images\tool2.png'); subplot(2,2,1); imshow(i); title('Image 1'); subplot(2,2,2); imshow(j); title('Image 2'); k=imabsdiff(i,j); subplot(2,2,3); imshow(k); title('Image 3 = Image 1 - Image 2');

|  | Image Processing Practical #2 |  |  |
| --- | --- | --- | --- |
| Name | Harshal Kadam | Roll Number | 21906A1024 |
| Subject/Course: | Image Processing |  |  |
| Topic | Image Enhancement |  |  |
| Image Negative |  |  |  |
| Code: figure(); i=imread("("C:\Users\student\Downloads \IPCV\images\balloons.png"); a=255-double(i); a=uint8(a); subplot(1,2,1); imshow(i); title("Original Image"); subplot(1,2,2); imshow(a); title("Negative Image"); Output: |  |  |  |
| Thresholding |  |  |  |
| Code: figure(); r=imread("("C:\Users\student\Downloads \IPCV\images\coins.png"); a=r; [row col]=size(a); |  |  |  |

T=input('Enter value of threshold:'); // here we took 100 for i=1:1:row for j=1:1:col if(r(i,j)>=T) a(i,j)=r(i,j); else a(i,j)=0; end end end subplot(1,2,1); imshow(r); title('Original Image'); subplot(1,2,2); imshow(a); title('Image obtained using threshold'); Output:

Log Transformation

Code: r=imread("("C:\Users\student\Downloads \IPCV\images\baboon.png") c=25; S=c*log(1+double(r)); O=uint8(S); subplot(2,2,1); imshow(r) title("Original Image"); subplot(2,2,2); imshow(O); title("Log Transformation")

Output:

Piece-Wise Linear Transformation

Code: a=imread("("C:\Users\student\Downloads \IPCV\images\peppers.png") subplot(2,2,1); imshow(a); title("Original Image") r=double(a)/255; c=1; gamma=8.3; s=c*(r).^gamma; subplot(2,2,2); imshow(s); title("Gamma Image") Output:

Gray Level Slicing

Code: Without Background a=imread("("C:\Users\student\Downloads \IPCV\images\peppers.png"); [r,c]=size(a); for i=1:r for j=1:c if (a(i,j)>50 & a(i,j)<100) x(i,j)=255; else x(i,j)=0; end end end x=uint8(x); subplot(2,2,1) imshow(a); title('Original Image'); subplot(2,2,2); imshow(x); Output: With Background a=imread(("C:\Users\student\Downloads \IPCV\images\coins.png"); [r,c]=size(a); for i=1:r for j=1:c if (a(i,j)>60 & a(i,j)<110) x(i,j)=255; else x(i,j)=a(i,j); // if without background use x(i,j)=0 end end end x=uint8(x); subplot(2,2,1) imshow(a); title('Original Image');

subplot(2,2,2); imshow(x); title('Grey level slicing without background'); Output:

Bit Plane Slicing

Code: f=imread("C:\Users\student\Downloads \IPCV\images\lena.png"); f=double(f); [r,c]=size(f);m=1; for i=1:r for j=1:c MSB(i,j)=bitand(f(i,j),128); seven(i,j)=bitand(f(i,j),64); sex(i,j)=bitand(f(i,j),32); five(i,j)=bitand(f(i,j),16); four(i,j)=bitand(f(i,j),8); three(i,j)=bitand(f(i,j),4); two(i,j)=bitand(f(i,j),2); LSB(i,j)=bitand(f(i,j),1); end end subplot(2,2,1);imshow(MSB);title(" MSB"); subplot(2,2,2);imshow(seven);title("7th"); subplot(2,2,3);imshow(six);title("6th"); subplot(2,2,4);imshow(five);title("5th"); subplot(2,2,5);imshow(four);title("4th"); subplot(2,2,6);imshow(three);title("3rd"); subplot(2,2,7);imshow(two);title("2nd"); subplot(2,2,8);imshow(LSB);title("1st"); subplot(2,2,9);imshow(f);title("original"); Output:

Original:

|  | Image Processing Practical #3 |  |  |
| --- | --- | --- | --- |
| Name | Harshal Kadam | Roll Number | 21306A1024 |
| Subject/Cours e: | Image Processing |  |  |
| Topic | Histogram Processing |  |  |
| Plot Histogram of an Image |  |  |  |

clear; a=imread("C:\Users\student\Downloads\seed.tif"); subplot(2,2,1); imshow(a); [r,c]=size(a); h=zeros(1,256); s=0:255 for i=1:r 3 for j=1:c 3 if (a(i,j)==0) a(i,j)=1; end k=a(i,j); h(k)=h(k)+1; end end subplot(2,2,2); plot(h); title('Histogram of the Image') Output:

Plot Histogram of Low Contrast, Bright , dark and High Contrast Images

clear; a=imread("C:\Users\student\Downloads\seed.tif"); subplot(4,2,1); title("Original Image") imshow(a); subplot(4,2,2); h1=imhist(a); plot(h1); subplot(4,2,3); a=a+100; title("hight Contrast Image") imshow(a); subplot(4,2,4)

h2=imhist(a); plot(h2); subplot(4,2,5); a=a-180; title("low Contrast Image") imshow(a); subplot(4,2,6) h3=imhist(a); plot(h3); mmin =min(a(:)); mmax = max(a(:)); lmin=0;lmax=255; //p1=(p-mmin)*(255/(mmax-mmin)); a=(a-mmin)*((lmax-lmin)/(mmax-mmin))+lmin; subplot(4,2,8); h4=imhist(a); plot(h4); subplot(4,2,7); title(""); imshow(a); Output:

Histogram Equalization

clc; clear; a=imread("C:\Program Files\scilab-6.0.2\IPCV\images\seed.tif"); [r,c]=size(a); subplot(2,2,1); imshow(a) title("Original Image") subplot(2,2,2); h=imhist(a); plot(h); title("Histogram of Original Image") a=double(a); big=max(max(a)); // largest intensity present in the image tot = r*c; // total no of pixels in the image z=zeros(1,256); //empty array for hostogram pdf=h/tot; // probabilty of occurance of intensity cdf(1)=pdf(1); //first cumulative addition for i=2:1:big cdf(i)=pdf(i)+cdf(i-1); end new=round(cdf*big); for i=1:1:r for j=1:1:c temp=a(i,j); if(temp==0) temp=1; end b(i,j)=new(temp); end end b=uint8(b); subplot(2,2,4); h9 =imhist(b) plot(h9); title('Equalized Histogram'); subplot(2,2,3) imshow(uint8(b)) title('Equalized mage'); OUTPUT:

|  | Image Processing Practical #4 |  |  |
| --- | --- | --- | --- |
| Name | Harshal Kadam | Roll Number | 21306A1024 |
| Subject/Course: | Image Processing |  |  |
| Topic | Image Smoothing in Spatial Domain |  |  |
| Averaging |  |  |  |

clc; clear; a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\moon.tif"); subplot(1,3,1); imshow(a); c=imnoise(a,'gaussian',0.3); d=double(c); b=d; //backup m=(1/9)*(ones(3,3)); //[1,1,1; 1,1,1; 1,1,1;] [r1,c1]=size(a); for i=2:r1-1 for j=2:c1-1 a1=d(i-1,j-1)+d(i-1,j)+d(i-1,j+1)+d(i,j- 1)+d(i,j)+d(i,j+1)+d(i+1,j)+d(i+1,j)+d(i+1,j+1); b(i,j)=a1*(1/9); end end subplot(1,3,1); imshow(a); title('Original Image'); subplot(1,3,2) imshow(uint8(b)); title ('Filtered Image'); subplot(1,3,3); imshow(uint8(c)); title('noisy image')

Min Filter

clc; clear; p1=imread("C:\Program Files\scilab-6.1.1\IPCV\images\moon.tif"); pmin=imnoise(p1,'salt & pepper'); //d=double(c); //b=d; //backup //m=(1/9)*(ones(3,3)); //[1,1,1; 1,1,1; 1,1,1;] figure; subplot(1,2,1); imshow(pmin); title('Noisy Image'); [r1,c1]=size(pmin); for i=2:1:r1-1 for j=2:1:c1-1 a1=[pmin(i-1,j-1) pmin(i-1,j) pmin(i-1,j+1) pmin(i,j-1) pmin(i,j) pmin(i,j+1) pmin(i+1,j-1) pmin(i+1,j) pmin(i+1,j+1)]; a2=min(min(a1)); newmin(i,j)=a2; end end subplot(1,2,2); imshow(newmin);

title('Image after min filtering');

Max Filter

clc; clear; figure; p1=imread("C:\Program Files\scilab-6.1.1\IPCV\images\moon.tif"); pmax=imnoise(p1,'salt & pepper'); //d=double(c); //b=d; //backup //m=(1/9)*(ones(3,3)); //[1,1,1; 1,1,1; 1,1,1;] figure; subplot(1,2,1); imshow(pmax); title('Noisy Image'); [r1,c1]=size(pmax); for i=2:1:r1-1 for j=2:1:c1-1 a1=[pmax(i-1,j-1) pmax(i-1,j) pmax(i-1,j+1) pmax(i,j-1) pmax(i+j) pmax(i,j+1) pmax(i+1,j-1) pmax(i+1,j) pmax(i+1,j+1)]; a2=max(max(a1)); newmax(i,j)=a2; end

end subplot(1,2,2); imshow(newmax); title('Image after max filtering');

Median Filter

clc; clear; p1=imread("C:\Program Files\scilab-6.1.1\IPCV\images\moon.tif"); pmed=imnoise(p1,'salt & pepper'); //d=double(c); //b=d; //backup //m=(1/9)*(ones(3,3)); //[1,1,1; 1,1,1; 1,1,1;] figure; subplot(1,2,1); imshow(pmed); title('Noisy Image'); [r1,c1]=size(pmed); for i=2:1:r1-1 for j=2:1:c1-1 a1=[pmed(i-1,j-1) pmed(i-1,j) pmed(i-1,j+1) pmed(i,j-1) pmed(i,j) pmed(i,j+1) pmed(i+1,j-1) pmed(i+1,j) pmed(i+1,j+1)]; a2=gsort(a1); med=a2(5);

newmed(i,j)=med;

end end

subplot(1,2,2); imshow(uint8(newmed));

title('Image after low_passmedian filtering');

Image Processing Practical #5

| Name | Harshal Kadam | Roll Number | 21306A1024 |
| --- | --- | --- | --- |
| Subject/Course: | Image Processing |  |  |
| Topic | Image Sharpening in Spatial Domain |  |  |

First Order Derivative Filter

clear; a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\morpex.png"); subplot(5,2,1) imshow(a); d=double(a); v=[1 0 -1;2 0 -2;1 0 -1]; h=[-1 -2 -1;0 0 0;1 2 1]; [r1,c1]=size(a); for i=2:(r1-1) for j=2:(c1-1) news(i,j) =(v(1)*d(i-1,j-1))+(v(2)*d(i-1,j))+(v(3)*d(i-1,j+1))+(v(4)*d(i,j- 1))+(v(5)*d(i,j))+(v(6)*d(i,j+1))+(v(7)*d(i+1,j-1))+(v(8)*d(i+1,j))+(v(9)*d(i+1,j+1)); end end subplot(5,2,2) imshow(uint8(news)) for i=2: (r1-1) for j=2: (c1-1) newh(i,j) =(h(1)*d(i-1,j-1))+(h(2)*d(i-1,j))+(h(3)*d(i-1,j+1))+(h(4)*d(i,j- 1))+(h(5)*d(i,j))+(h(6)*d(i,j+1))+(h(7)*d(i+1,j-1))+(h(8)*d(i+1,j))+(h(9)*d(i+1,j+1)); end end subplot(5,2,3) imshow(uint8(newh)) for i=2:(r1-1) for j=2:(c1-1) newvr(i,j) =(v(1)*-1*d(i-1,j-1))+(v(2)*-1*d(i-1,j))+(v(3)*-1*d(i-1,j+1))+(v(4)*- 1*d(i,j-1))+(v(5)*-1*d(i,j))+(v(6)*-1*d(i,j+1))+(v(7)*-1*d(i+1,j-1))+(v(8)*- 1*d(i+1,j))+(v(9)*-1*d(i+1,j+1)); end end subplot(5,2,4) imshow(uint8(newvr)) for i=2: (r1-1) for j=2: (c1-1) newrh(i,j) =(h(1)*-1*d(i-1,j-1))+(h(2)*-1*d(i-1,j))+(h(3)*-1*d(i-1,j+1))+(h(4)*- 1*d(i,j-1))+(h(5)*-1*d(i,j))+(h(6)*-1*d(i,j+1))+(h(7)*-1*d(i+1,j-1))+(h(8)*- 1*d(i+1,j))+(h(9)*-1*d(i+1,j+1)); end end subplot(5,2,5)

imshow(uint8(newrh)); final=uint8(news)¦uint8(newh)¦uint8(newvr)¦uint8(newrh); subplot(5,2,6) imshow(uint8(final)) Output:

Second Order Derivative – Laplacian Filter

clear; p=imread(' C:\Program Files\scilab-6.0.1\IPCV\images \morpex.png') subplot(1,2,1) title('Original image') imshow(p) d=double(p) v=[0 1 0;1 -4 1;0 1 0]; [r1,c1]=size(p); for i=2:1:r1-1 for j=2:1:c1-1 newv(i,j)=(v(1)*d(i-1,j-1))+(v(2)*d(i-1,j))+(v(3)*d(i-1,j+1))+(v(4)*d(i,j- 1))+(v(5)*d(i,j))+(v(6)*d(i,j+1))+(v(7)*d(i+1,j-1))+(v(8)*d(i+1,j))+(v(9)*d(i+1,j+1)) end end

subplot(1,2,2)

imshow(newv)

Output:

Image Processing Practical #6

| Name | Harshal Kadam | Roll Number | 21306A1024 |
| --- | --- | --- | --- |
| Subject/Course: | Image Processing |  |  |
| Topic | Image Smoothing in Frequency Domain |  |  |

Ideal Low Pass Filter

clc; a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\camera man.png"); a1=double(a); [r,c]=size(a1); d=input("Enter the cut off frequency"); for u=1:r for v= 1:c d1=(((u-(r/2))^2)+ ((v-(c/2))^2))^0.5 if d1<=d h(u,v)=1; else h(u,v)=0; end end end b=fft2(a1); c=fftshift(b); new=h.*c; new1=abs(ifft(new)); subplot(2,2,1); imshow(uint8(a1)); subplot(2,2,2); imshow(uint8(new1)); Output:

Butterworth Low Pass Filter

clc; a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\camera man.png"); a1=double(a); [r,c]=size(a1); d=input("Enter the cut off frequency"); n=input("Enter exponential"); for u=1:r for v= 1:c d1=(((u-(r/2))^2)+ ((v-(c/2))^2))^0.5; h(u,v)=1/(1+(d1/d)^(2*n)); end end

b=fft2(a1); c=fftshift(b); new=h.*c; new1=abs(ifft(new)); subplot(2,2,1); imshow(uint8(a1)); subplot(2,2,2); imshow(uint8(new1)); Output:

Gaussian Low Pass Filter

clc; a=imread(" C:\Program Files\scilab-6.0.1\IPCV\images\camera man.png"); a1=double(a); [r,c]=size(a1); d=input("Enter the cut off frequency"); for u=1:r for v= 1:c d1=(((u-(r/2))^2)+ ((v-(c/2))^2))^0.5 h(u,v)=exp(-(d1*d1)/(2*d*d)); end end b=fft2(a1); c=fftshift(b); new=h.*c; new1=abs(ifft(new)); subplot(2,2,1); imshow(uint8(a1)); subplot(2,2,2); imshow(uint8(new1)); Output:

Image Processing Practical #7

| Name | Harshal Kadam | Roll Number | 21306A1024 |
| --- | --- | --- | --- |
| Subject/Course: | Image Processing |  |  |
| Topic | Image Sharpening in Frequency Domain |  |  |

clc;

a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\camera man.png"); a1=double(a);

[r,c]=size(a1);

d=input("Enter the cut off frequency"); for u=1:r

for v= 1:c

d1=(((u-(r/2))^2)+ ((v-(c/2))^2))^0.5

if d1<=d

h(u,v)=0; else

h(u,v)=1;

end end end

b=fft2(a1); c=fftshift(b); new=h.*c; new1=abs(ifft(new)); subplot(2,2,1); imshow(uint8(a1)); subplot(2,2,2); imshow(uint8(new1));

Output:

Ideal High Pass Filter

Butterworth High Pass Filter

clc; a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\camera man.png"); a1=double(a); [r,c]=size(a1); d=input("Enter the cut off frequency"); n=input("Enter exponential"); for u=1:r for v= 1:c d1=(((u-(r/2))^2)+ ((v-(c/2))^2))^0.5; h(u,v)=1/(1+(d/d1)^(2*n)); end end b=fft2(a1); c=fftshift(b); new=h.*c; new1=abs(ifft(new)); subplot(2,2,1); imshow(uint8(a1)); subplot(2,2,2); imshow(uint8(new1)); Output:

Gaussian High Pass Filter

clc; a=imread(" C:\Program Files\scilab-6.0.1\IPCV\images\camera man.png"); a1=double(a); [r,c]=size(a1); d=input("Enter the cut off frequency"); for u=1:r for v= 1:c d1=(((u-(r/2))^2)+ ((v-(c/2))^2))^0.5 h(u,v)=1- exp(-(d1*d1)/(2*d*d)); end end b=fft2(a1); c=fftshift(b); new=h.*c; new1=abs(ifft(new)); subplot(2,2,1); imshow(uint8(a1)); subplot(2,2,2); imshow(uint8(new1)); Output:

Image Processing Practical #8

| Name | Harshal Kadam | Roll Number | 21306A1024 |
| --- | --- | --- | --- |
| Subject/Course: | Image Processing |  |  |
| Topic | Image Restoration |  |  |

Arithmetic Mean Filter to Remove Gaussian Noise

Code:

clc;

clear all;

a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\moon.tif"); figure;

c=imnoise(a,'gaussian'); d=double(c);

b=d; //backup [r1,c1]=size(a); for i=2:r1-1

for j=2:c1-1

a1=d(i-1,j-1)+d(i-1,j)+d(i-1,j+1)+d(i,j-1)+d(i,j)+d(i,j+1)+d(i+1,j-1)+d(i+1,j)+d(i+1,j+1); p=a1*(1/9);

b(i,j)=p; end

end subplot(1,3,1); imshow(a);

title('Original Image'); subplot(1,3,2); imshow(c);

title('Noisy Image'); imshow(uint8(c)); subplot(1,3,3)

title("Arithmethic Mean Filter") imshow(uint8(b));

Output:

Geometric Mean Filter to Remove Gaussian Noise

Code: clc; clear all; a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\moon.tif"); figure; c=imnoise(a,'gaussian'); d=double(c); b=d; //backup [r1,c1]=size(a); for i=2:r1-1 for j=2:c1-1 a1=d(i-1,j-1)*d(i-1,j)*d(i-1,j+1)*d(i,j-1)*d(i,j)*d(i,j+1)*d(i+1,j-1)*d(i+1,j)*d(i+1,j+1); p=a1.^(1/9); b(i,j)=p; end end subplot(1,3,1); imshow(a); title('Original Image'); subplot(1,3,2); imshow(c); title('Noisy Image'); imshow(uint8(c)); subplot(1,3,3) title("Geometric Mean Filter") imshow(uint8(b)); Output:

Harmonic Mean Filter to remove Salt & Pepper Noise

Code: clc; clear all; a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\moon.tif"); figure; c=imnoise(a,'gaussian'); d=double(c); b=d; //backup [r1,c1]=size(a); for i=2:r1-1 for j=2:c1-1 a1=1/d(i-1,j-1)+1/d(i-1,j)+1/d(i-1,j+1)+1/d(i,j-1)+1/d(i,j)+1/d(i,j+1)+1/d(i+1,j- 1)+1/d(i+1,j)+1/d(i+1,j+1); b(i,j)=9/a1; end end subplot(1,3,1); imshow(a); title('Original Image'); subplot(1,3,2); imshow(c); title('Noisy Image'); imshow(uint8(c)); subplot(1,3,3) title('Harmonic Mean Filter') imshow(uint8(b)); Output:

Contra Harmonic Mean Filter to Remove Salt & Pepper Noise

Code: clc; clear all; a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\moon.tif"); figure; q=input("enter the q"); c=imnoise(a,'salt & pepper',0.007); d=double(c); b=d; //backup [r1,c1]=size(a); for i=2:r1-1 for j=2:c1-1 a1=d(i-1,j-1)^(q+1)+d(i-1,j)^(q+1)+d(i-1,j+1)^(q+1)+d(i,j- 1)^(q+1)+d(i,j)^(q+1)+d(i,j+1)^(q+1)+d(i+1,j-1)^(q+1)+d(i+1,j)^(q+1)+d(i+1,j+1)^(q+1); a2=d(i-1,j-1)^(q)+d(i-1,j)^(q)+d(i-1,j+1)^(q)+d(i,j-1)^(q)+d(i,j)^(q)+d(i,j+1)^(q)+d(i+1,j- 1)^(q)+d(i+1,j)^(q)+d(i+1,j+1)^(q); b(i,j)=a1/a2; end end subplot(1,3,1); imshow(a); title('Original Image'); subplot(1,3,2); imshow(c); title('Noisy Image'); imshow(uint8(c)); subplot(1,3,3) title('Harmonic Mean Filter') imshow(uint8(b)); Output:

Mid Point Filter to Remove Salt & Pepper Noise

clc; clear all; a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\moon.tif"); figure; c=imnoise(a,'salt & pepper',0.008); d=double(c); b=d; //backup [r1,c1]=size(a); for i=2:r1-1 for j=2:c1-1 a1=[d(i-1,j-1) d(i-1,j) d(i-1,j+1) d(i,j-1) d(i,j) d(i,j+1) d(i+1,j-1) d(i+1,j) d(i+1,j+1)]; a2=gsort(a1); maxi=a2(9); mini=a2(1); mid=(1/2)*(mini+maxi); b(i,j)=mid; end end b=uint8(b); subplot(1,3,1); imshow(a); title('Original Image'); subplot(1,3,2); imshow(c); title('Noisy Image'); imshow(uint8(c)); subplot(1,3,3)

title('MidPoint Filter') imshow(uint8(b)); Output:

Image Processing Practical #9

| Name | Harshal Kadam | Roll Number | 21306A1024 |
| --- | --- | --- | --- |
| Subject/Course: | Image Processing |  |  |
| Topic | Color Image Processing |  |  |

Display different color planes and plot histogram of color image

figure;

a=imread("("C:\Users\student\Downloads \IPCV\images\peppers.png"); R=a(:,:,1);

G=a(:,:,2);

B=a(:,:,3);

subplot(3,2,1); imshow(a); title("Origninal Image"); subplot(3,2,2); imshow(R);

title("Red Component");

subplot(3,2,3); imshow(G); title("Green Component"); subplot(3,2,4); imshow(B); title("Blue component"); [yR,x]=imhist(R); [yG,x]=imhist(G); [yB,x]=imhist(B); subplot(3,2,5); plot(x,yR,x,yG,x,yB,"Linewidth",2); //subplot(3,2,5); xlabel("RGB Intensity"); ylabel("No.of Pixels"); set(gca(),"grid",[1,1]) Output:

Convert RGB image to HSV and CMY model

figure; a=imread("("C:\Users\student\Downloads \IPCV\images\peppers.png"); HSV=rgb2hsv(a); subplot(2,1,1 title("RGB2HSV") imshow(HSV); CMY=imcomplement(a); subplot(2,1,2);

title("RGB2CMY") imshow(CMY); Output:

Color Image Smoothing

clc;clear all; I=imread("C:\Program Files\scilab-6.0.1\IPCV\images\peppers.png"); F = fspecial('average',11); pic(:,:,1) = imfilter(I(:,:,1),F); pic(:,:,2) = imfilter(I(:,:,2),F); pic(:,:,3) = imfilter(I(:,:,3),F); Average_Image = uint8(pic); subplot(2,2,1); imshow(I); title('Original Image'); subplot(2,2,2); imshow(Average_Image); title('Average Filtered Image'); I=imnoise(I,'gaussian',0.07); F1 = fspecial('gaussian',3); pic1(:,:,1) = imfilter(I(:,:,1),F1);pic1(:,:,2) = imfilter(I(:,:,2),F1); pic1(:,:,3) = imfilter(I(:,:,3),F1); GLPF_Image = uint8(pic1); subplot(2,2,3); imshow(I); title('Noisy Image(Gaussian Noise)'); subplot(2,2,4); imshow(GLPF_Image); title('GLPF Filtered Image') Output:-

Color Image Sharpening

clc;clear all; I=imread("C:\Program Files\scilab-6.0.1\IPCV\images\peppers.png"); F=fspecial('prewitt'); pic(:,:,1) = imfilter(I(:,:,1),F); pic(:,:,2) = imfilter(I(:,:,2),F); pic(:,:,3) = imfilter(I(:,:,3),F); Sharp_Image = uint8(pic);s ubplot(1,2,1); imshow(I); title('Original Image'); subplot(1,2,2); imshow(Sharp_Image); title('Sharp Filtered Image') Output:-

|  | Image Processing Practical #10 |  |  |
| --- | --- | --- | --- |
| Name | Harshal Kadam | Roll Number | 21306A1024 |
| Subject/Course: | Image Processing |  |  |
| Topic | Morphological Image Processing |  |  |
| Erosion |  |  |  |

Code:- //Program to apply erosion clc; clear all; figure; a=imread('C:\Program Files\scilab-6.0.1\IPCV\images\morpex.png'); d=a; [r,c]=size(d); m=ones(3,3); for i=2:1:r-1 for j=2:1:c-1 new=[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A1(i,j)=min(new); end end subplot(1,2,1); imshow(a); title('Original Image'); subplot(1,2,2); imshow(A1); title('Erosion Image'); Output:-

dilation

Code:- //Program to apply dilation clc; clear all; figure; a=imread('C:\Program Files\scilab-6.0.1\IPCV\images\morpex.png'); d=a; [r,c]=size(d); m=ones(3,3); for i=2:1:r-1 for j=2:1:c-1 new=[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A1(i,j)=max(new); end end subplot(1,2,1); imshow(a); title('Original Image'); subplot(1,2,2);

imshow(A1); title('Dilation Image'); Output:

Opening

figure; a=imread('C:\Program Files\scilab-6.0.1\IPCV\images\line.tif'); d=a; [r,c]=size(d); m=ones(3,3); for i=2:1:r-1 for j=2:1:c-1 new=[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A1(i,j)=min(new); end end d=A1; [r,c]=size(d); for i=2:1:r-1 for j=2:1:c-1 new=[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A2(i,j)=max(new);

end end subplot(1,2,1); imshow(a); title('Original Image'); subplot(1,2,2); imshow(A2); title('processed image - o'); Output:-

Closing

clc; clear all; figure; a=imread('C:\Program Files\scilab-6.0.1\IPCV\images\circbw.png'); d=a; [r,c]=size(d); m=ones(3,3); for i=2:1:r-1 for j=2:1:c-1 new=[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A1(i,j)=max(new); end end d=A1; [r,c]=size(d); for i=2:1:r-1

for j=2:1:c-1 new=[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A2(i,j)=min(new); end end subplot(1,2,1); imshow(a); title('Original Image'); subplot(1,2,2); imshow(A2); title('processed image - o'); Output:-

Boundary Extraction

clc; clear a= imread('C:\Program Files\scilab-6.0.1\IPCV\images\boundary.tif'); d=a; [r,c]=size(d); m= ones(3,3) for i=2:1:r-1 for j=2:1:c-1

new =[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A1(i,j)=min(new); aa(i,j)= d(i,j) - A1(i,j) end end subplot(1,2,1) imshow(a); title("Original Image") subplot(1,2,2) imshow(aa) title("Processed Image Boundary Extraction")

Morphological Gradient

clc; Clear a= imread('C:\Program Files\scilab-6.0.1\IPCV\images\boundary.tif'); d=a; [r,c]=size(d); m= ones(3,3) for i=2:1:r-1 for j=2:1:c-1

new =[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))] A1(i,j)=max(new); End end d=A1; [r,c]= size(d) for i=2:1:r-1 for j=2:1:c-1 new =[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))] A2(i,j)=min(new); aa(i,j)= A1(i,j)-A2(i,j) end end subplot(1,2,1) imshow(a); title("Original Image") subplot(1,2,2) imshow(aa) title("Processed Image Gradient")

Image Processing Practical #11

| Name | Harshal Kadam | Roll Number | 21306A1024 |
| --- | --- | --- | --- |
| Subject/Course: | Image Processing |  |  |
| Topic | Image Segmentation |  |  |

Edge Detection – Sobel operator

clc;

clear all;

p=imread('C:\Program Files\scilab-6.0.1\IPCV\images\edge2.tif'); figure;

subplot(3,2,1); imshow(p); title('Original Image');

v1=[1,0,-1;2, 0, -2;1, 0, -1]; //x-direction

h1=[-1, -2, -1; 0, 0 ,0; 1, 2, 1 ]; //y-direction

v2=[-1, 0, 1; -2, 0, 2; -1, 0, 1];

h2=[1, 2, 1; 0, 0, 0; -1, -2 ,-1];

gv1=abs(imfilter(double(p),v1)); subplot(3,2,2);

imshow(gv1); title('Verical Edges');

gh2=abs(imfilter(double(p),h2)); gh1=abs(imfilter(double(p),h1)); subplot(3,2,3);

imshow(gh1); subplot(3,2,4); imshow(gh2); title('horizontal Edges');

gv2=abs(imfilter(double(p),v2)); subplot(3,2,5);

imshow(gv2); title('vertical2 Edges'); finaledge=gh1+gv2+gh2+gv1; subplot(3,2,6); imshow(finaledge); title('Final Image');

Output:

Edge Detection – Canny Algorithm

clc; clear all; p=imread('C:\Program Files\scilab-6.0.1\IPCV\images\edge2.tif'); figure; subplot(2,2,1); imshow(p); title('Original Image'); d=double(p); thresh=0.02; sigma=3; // 3, 5, 7 E=edge(d, 'canny', thresh, sigma); subplot(2,2,2); imshow(E); title('Canny Edge Image'); Output: