Image Processing

| Name | GORIMA SAYED | Roll Number | 21306A1054 |
| --- | --- | --- | --- |
| Subject/Cour se: | Image Processing |  |  |
| Topic | Image Processing Basics |  |  |

Pixel

Program to calculate number of samples required for an image /Programtocalculatenumberofsamplesrequiredforanimage figure; m=4; n=6; N=400; Fs=m*N*n*N; disp(Fs,'Number of samples required to preserve the information in the image=');

Image Properties

Program to accessimageproperties– Dimension, height, width, number of channels, accessing and modifying anypixel /Programtoaccessimageproperties–Dimension,height,width,numberof channels,accessingandmodifyinganypixel figure; i=imread("C:\Program Files\scilab 6.0.2\IPCV\images\lena.png"); s=size(i); disp(s,"Dimensions"); disp(s(1),"height"); disp(s(2),"width"); disp(s(3),"No. of Channels"); disp(i(100,100,3)); i(100,100,3)=1; disp(i(100,100,3));

Sampling

Program to study the effects of reducing the spatial resolution of a digital image /Programtostudytheefectsofreducingthespatialresolutionofadigitalimage figure; i=imread('C:\Program Files\scilab-6.0.2\IPCV\images\lena_dark.png'); disp("size of original image",size(i)); subplot(3,3,1); imshow(i); title('original image'); j1=imresize(i,0.8); disp("size of resized Image",size(j1)); subplot(3,3,2); imshow(j1); title('resized image 0.8');

j2=imresize(i,0.5); disp("size of resized Image",size(j2)); subplot(3,3,3);imshow(j2); title('resized image 0.5')

Quantization

Program to study the effects of varying the number of intensity levels in a digital image /Programtostudytheefectsofvaryingthenumberofintensitylevelsinadigital imagefigure; i=imread('C:\Program Files\scilab-6.0.2\IPCV\images\lena_dark.png'); subplot(2,2,1); imshow(i); title('original image') i=double(i); k1=(i*255)/64; subplot(2,2,2); ––k1=uint8(k1); imshow(uint8(k1)); title('Quantization 64'); k2=(i*255)/32;

subplot(2,2,3); k2=uint8(k2); imshow(uint8(k2)); title('Quantization 32'); k3=(i*255)/16; subplot(2,2,4); k3=uint8(k3); imshow(uint8(k3)); title('Quantization 16');

Image Addition

Program to perform image addition for noise reduction. /Programtoperformimageadditionfornoisereduction. figure; i=imread('C:\Program Files\scilab-6.0.2\IPCV\images\noisy.png'); i=imnoise(i,'salt & pepper',0.001);

subplot(1,2,1); imshow(i); title('noisy Image'); k=imadd(i,50); –––subplot(1,2,2); imshow(k); title('Smooth Image after performing image addition');

Image Subtraction

Program to compare images using subtraction for enhancing the difference between images /Programtocompareimagesusingsubtractionforenhancingthediference betweenimages figure; i=imread('C:\Program Files\scilab-6.0.2\IPCV\images\tool1.png'); j=imread('C:\Program Files\scilab-6.0.2\IPCV\images\tool2.png'); subplot(2,2,1); imshow(i); title('Image 1'); subplot(2,2,2); imshow(j); title('Image 2'); k=imabsdiff(i,j); subplot(2,2,3); imshow(k); title('Image 3 = Image 1 - Image 2');

Image Processing Practical #2

| Name | GORIMA SAYED | Roll Number | 21306A1054 |
| --- | --- | --- | --- |
| Subject/Cours e: | Image Processing |  |  |
| Topic | Image Enhancement |  |  |

Image Negative

Code: //ImageNegatvie figure; r=imread('C:\Program Files\scilab-6.0.2\IPCV\images\checkerbox.png'); subplot(2,2,1); imshow(r); title('original Image'); L=2^8; neg=(L-1)-r; subplot(2,2,2); imshow(neg); title('Negative Image'); Output: -

Vidyalankar School of Information Technology

Thresholding

Code: figure; r=imread('C:\Program Files\scilab-6.0.2\IPCV\images\blurry_moon.tif'); a=r; [row col]=size(a); T=input('Enter value of threshold:'); // valueofthreshold= 70 for i=1:1:row for j=1:1:col if(r(i,j)>=T) a(i,j)=r(i,j); else a(i,j)=0; end end end subplot(1,2,1); imshow(r); title('Original Image'); subplot(1,2,2); imshow(a); title('Image obtained using threshold');

Output:-

Log Transformation

Code:- figure; r=imread('C:\Program Files\scilab-6.0.2\IPCV\images\log.tif'); c=15; S=c*log(1+double(r)); O=uint8(S); subplot(2,2,1); imshow(r) title("Original Image"); subplot(2,2,2); imshow(O); title("Log Transformation") Output:-

Piece-Wise Linear Transformation

Code:- /powerlawtransformation figure; a = imread('C:\Program Files\scilab-6.0.2\IPCV\images\blurry_moon.tif'); subplot(2,2,1); imshow(a); title("Original Image") r=double(a)/255; c=1; gamma=0.3; s=c*(r).^gamma; subplot(2,2,2); imshow(s); title("Gamma Image") Output:-

Gray Level Slicing

With Background Code:- /graylevelslicing figure; p=imread('C:\Program Files\scilab-6.0.2\IPCV\images\log.tif'); z=double(p); [row col]=size(p); for i=1:1:row for j=1:1:col if(z(i,j)>50)&&(z(i,j)<200) z(i,j)=150; else z(i,j)=p(i,j); end end end subplot(1,2,1); imshow(p); title('Original Image'); subplot(1,2,2); imshow(z); title('Grey level slicing with background'); Output:-

Without Background Code:- /graylevelslicing figure; p=imread('C:\Program Files\scilab-6.0.2\IPCV\images\log.tif'); z=double(p); [row col]=size(p); for i=1:1:row for j=1:1:col if(z(i,j)>50)&&(z(i,j)<100) z(i,j)=200; else z(i,j)=20 end end end subplot(1,2,1); imshow(p); title('Original Image'); subplot(1,2,2); imshow(z); title('Grey level slicing without background'); Output:-

Bit Plane Slicing

Code:- figure; B=imread('C:\Program Files\scilab-6.0.2\IPCV\images\coins.png'); [r,c]=size(B); for i=1:r for j=1:c MSB(i,j)=bitand(B(i,j),bin2dec('10000000')); LSB(i,j)=bitand(B(i,j),bin2dec('00000001')); Second(i,j)=bitand(B(i,j),bin2dec('01000000')); Third(i,j)=bitand(B(i,j),bin2dec('00100000')); Fourth(i,j)=bitand(B(i,j),bin2dec('00010000')); Fifth(i,j)=bitand(B(i,j),bin2dec('00001000')); Sixth(i,j)=bitand(B(i,j),bin2dec('00000100')); Seventh(i,j)=bitand(B(i,j),bin2dec('00000010')); end end subplot(4,4,1); imshow(MSB); title("Bit Plane 7") subplot(4,4,2); imshow(Second); title("Bit Plane 6") subplot(4,4,3); imshow(Third); title("Bit Plane 5") subplot(4,4,4); imshow(Fourth); title("Bit Plane 4") subplot(4,4,5); imshow(Fifth); title("Bit Plane 3") subplot(4,4,6); imshow(Sixth); title("Bit Plane 2") subplot(4,4,7); imshow(Seventh); title("Bit Plane 1") subplot(4,4,8); imshow(LSB); title("Bit Plane 0") subplot(4,4,9); imshow(B); title("Original Image") Output:-

Image Processing

| Name | GORIMA SAYED | Roll Number | 21306A1054 |
| --- | --- | --- | --- |
| Subject/Cour se: | Image Processing |  |  |
| Topic | Histogram Processing |  |  |

Plot Histogram of an Image

Code:- figure; img=imread("C:\Program Files\scilab-6.0.2\IPCV\images\Lena_dark.png") subplot(2,2,1) title('Original Image') imshow(img) h1=imhist(img); subplot(2,2,2); plot(h1) title('Predefined Histogram,'); [r,c]=size(img); h=zeros(1,256); s=0:255 for i=1:r for j=1:c if(img(i,j)==0) img(i,j)=1 end k=img(i,j); h(k)=h(k)+1; end end

subplot(2,2,3); plot(h); title('Self defined logic'); Output:-

Plot Histogram of Low Contrast, Bright , dark and High Contrast Images

Code:- clear all; img=imread("C:\Program Files\scilab-6.0.2\IPCV\images\seed.tif") subplot(4,2,1) title('Original Image') imshow(img) h1=imhist(img); subplot(4,2,2); plot(h1) title('Original Histogram');

darkimg=img darkimg=darkimg-80 subplot(4,2,3) imshow(darkimg) h2=imhist(darkimg) subplot(4,2,4) plot(h2) title("Histogram of dark image") brightimg=img brightimg=brightimg+100 subplot(4,2,5) imshow(brightimg) h3=imhist(brightimg) subplot(4,2,6) plot(h3) title("Histogram of bright image") a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\seed.tif"); mmin =min(a(:)); mmax = max(a(:)); lmin=0; lmax=255; a1=(a-mmin)*((lmax-lmin)/(mmax-mmin))+lmin; subplot(4,2,7) imshow(a1) h5=imhist(a1) subplot(4,2,8) plot(h5) Output:-

Histogram Equalization

Image Processing Practical #4

| Name | GORIMA SAYED | Roll Number | 21306A1054 |
| --- | --- | --- | --- |
| Subject/Cours e: | Image Processing |  |  |
| Topic | Image Smoothing in Spatial Domain |  |  |

Averaging

Code:- clc; figure; a=imread("C:\Program Files\scilab-6.0.1\IPCV-4.1.2-win64- bin\IPCV\images\blurry_moon.tif") subplot(1,3,1) title("Original image") imshow(a) //Adding noise c=imnoise(a,'gaussian') subplot(1,3,2) title("Noisy image") imshow(c) d=double(c); b=d; m=(1/9)*(ones(3,3)); [r1,c1]=size(a); for i=2;r1-1 for j=2:c1-1 a1=d(i-1,j-1)+d(i-1,j)+d(i-1,j+1)+d(i,j-1)+d(i,j)+d(i,j+1)+d(i+1,j- 1)+d(i+1,j)+d(i+1,j+1);

b(i,j)=a1*(1/9); end end subplot(1,3,3) title('Filtered Image') imshow(uint8(b)) Output:-

Min Filter

Code:- /min clc; figure; p1=imread("C:\Program Files\scilab-6.0.2\IPCV\images\blurry_moon.tif"); subplot(1,3,1) title("Original image") imshow(p1) pmin=imnoise(p1,'salt & pepper',0.2); subplot(1,3,2);

imshow(pmin); title('Noisy Image'); [r1,c1]=size(pmin); for i=2:1:r1-1 for j=2:1:c1-1 a1=[pmin(i-1,j-1) pmin(i-1,j) pmin(i-1,j+1) pmin(i,j-1) pmin(i,j) pmin(i,j+1) pmin(i+1,j-1) pmin(i+1,j) pmin(i+1,j+1)]; a2=min(min(a1)); newmin(i,j)=a2; end end subplot(1,3,3); imshow(newmin); title('Image aftermin filtering'); Output:-

Max Filter

Code:-

/max clc; figure;

p1=imread("C:\Program Files\scilab-6.0.2\IPCV\images\blurry_moon.tif"); subplot(1,3,1)

title("Original image") imshow(p1)

pmax=imnoise(p1,'salt & pepper',0.03); subplot(1,3,2);

imshow(pmax); title('Noisy Image');

[r1,c1]=size(pmax); for i=2:1:r1-1

for j=2:1:c1-1

a1=[pmax(i-1,j-1) pmax(i-1,j) pmax(i-1,j+1) pmax(i,j-1) pmax(i,j) pmax(i,j+1) pmax(i+1,j-1) pmax(i+1,j) pmax(i+1,j+1)];

a2=max(max(a1)); newmax(i,j)=a2;

end end

subplot(1,3,3); imshow(newmax);

title('Imageaftermaxfiltering'); Output:-

Median Filter

Code:- /med figure; p1=imread("C:\Program Files\scilab-6.0.2\IPCV\images\blurry_moon.tif"); pmed=imnoise(p1,'salt & pepper'); subplot(1,2,1); imshow(pmed); title('Noisy Image');

[r1,c1]=size(pmed); for i=2:1:r1-1 for j=2:1:c1-1 a1=[pmed(i-1,j-1) pmed(i-1,j) pmed(i-1,j+1) pmed(i,j-1) pmed(i,j) pmed(i,j+1) pmed(i+1,j-1) pmed(i+1,j) pmed(i+1,j+1)]; a2=gsort(a1); med=a2(5); newmed(i,j)=med; end end subplot(1,2,2); imshow(newmed); title('Imageaftermedfiltering'); Output:-

Image Processing Practical #5

| Name | GORIMA SAYED | Roll Number | 21306A1054 |
| --- | --- | --- | --- |
| Subject/Cours e: | Image Processing |  |  |
| Topic | Image Sharpening in Spatial Domain |  |  |

First Order Derivative Filter

Code:- //5-a figure; p=imread("C:\Program Files\scilab-6.0.1\IPCV\images\morpex.png"); subplot(3,2,1); imshow(p); title("Original Image"); d=double(p); v=[1 0 -1;2 0 -2;1 0 -1]; h=[-1 -2 -1; 0 0 0 ; 1 2 1]; [r1,c1]=size(p); for i=2:1:r1-1 for j=2:1:c1-1 newv(i,j)=(v(1)*d(i-1,j-1))+(v(2)*d(i-1,j))+(v(3)*d(i-1,j+1))+(v(4)*d(i,j- 1))+(v(5)*d(i,j))+(v(6)*d(i,j+1))+(v(7)*d(i+1,j+1))+(v(8)*d(i+1,j))+(v(9)*d(i+1,j+1)); end end subplot(3,2,2); imshow(uint8(newv)); title("Image after applying vertical sobel operator"); for i=2:1:r1-1 for j=2:1:c1-1 newh(i,j)=(h(1)*d(i-1,j-1))+(h(2)*d(i-1,j))+(h(3)*d(i-1,j+1))+(h(4)*d(i,j-

1))+(h(5)*d(i,j))+(h(6)*d(i,j+1))+(h(7)*d(i+1,j+1))+(h(8)*d(i+1,j))+(h(9)*d(i+1,j+1)); end end subplot(3,2,3); imshow(uint8(newh)); title("Image after applying horizontal sobel operator"); v1=[-1 0 1; -2 0 2; -1 0 1]; h1=[1 2 1; 0 0 0; -1 -2 -1]; for i=2:1:r1-1 for j=2:1:c1-1 newv1(i,j)=(v1(1)*d(i-1,j-1))+(v1(2)*d(i-1,j))+(v1(3)*d(i-1,j+1))+(v1(4)*d(i,j- 1))+(v1(5)*d(i,j))+(v1(6)*d(i,j+1))+(v1(7)*d(i+1,j+1))+(v1(8)*d(i+1,j))+(v1(9)*d(i+1,j+1)); end end subplot(3,2,4); imshow(uint8(newv1)); title("Image after applying vertical sobel operator rotated 180 degree"); for i=2:1:r1-1 for j=2:1:c1-1 newh1(i,j)=(h1(1)*d(i-1,j-1))+(h1(2)*d(i-1,j))+(h1(3)*d(i-1,j+1))+(h1(4)*d(i,j- 1))+(h1(5)*d(i,j))+(h1(6)*d(i,j+1))+(h1(7)*d(i+1,j+1))+(h1(8)*d(i+1,j))+(h1(9)*d(i+1,j+1)); end end subplot(3,2,5); imshow(uint8(newh1)); title("Image after applying horizontal sobel operator rotated 180 degree"); finalimg=uint8(newv) ¦ uint8(newh) ¦ uint8(newv1) ¦ uint8(newh1); subplot(3,2,6); imshow(finalimg); title("Final Image"); Output:-

Second Order Derivative – Laplacian Filter

Code:- figure; p=imread('C:\Program Files\scilab-6.0.1\IPCV\images\morpex.png') subplot(1,2,1) title('Original image') imshow(p) d=double(p) v=[0 1 0;1 -4 1;0 1 0]; [r1,c1]=size(p); for i=2:1:r1-1 for j=2:1:c1-1 newv(i,j)=(v(1)*d(i-1,j-1))+(v(2)*d(i-1,j))+(v(3)*d(i-1,j+1))+(v(4)*d(i,j- 1))+(v(5)*d(i,j))+(v(6)*d(i,j+1))+(v(7)*d(i+1,j-1))+(v(8)*d(i+1,j))+(v(9)*d(i+1,j+1)) end end

subplot(1,2,2) imshow(newv) Output:-

Image Processing Practical #6

| Name | GORIMA SAYED | Roll Number | 21306A1054 |
| --- | --- | --- | --- |
| Subject/Cours e: | Image Processing |  |  |
| Topic | Image Smoothing in Frequency Domain |  |  |

Ideal Low Pass Filter

Code:- /IdealLowPassFilter figure; p=imread("C:\Program Files\scilab-6.0.1\IPCV\images\cameraman.tif"); a1=double(p); r=size(a1,1); c=size(a1,2); d0=input('Enter the cut-off frequency-(Radius):-'); for u=1:1:r for v=1:1:c d=(((u-(r/2))^2)+((v-(c/2))^2))^0.5; if d<=d0 h(u,v)=1; else h(u,v)=0; end end end b=fft2(a1); c=fftshift(b); new=h.*c; new1=abs(ifft(new)); subplot(1,2,1);

imshow(uint8(p)); title('Original Image'); subplot(1,2,2); imshow(uint8(new1)); title(['Filtered Image with radius = ', string(d0)]); Output:-

Butterworth Low Pass Filter

Code:- /Buterworthlowpassfilter figure; a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\cameraman.tif"); a1=double(a); r=size(a1,1); c=size(a1,2); d0=input('enter the cutt-of frequency-(Radius):-'); n=input("enter exponential") foru=1:1:r forv=1:1:c d=(((u-(r/2))^2)+ ((v-(c/2))^2))^0.5; h(u,v)=1/(1+((d/d0)^(2*n))); end end b=fft2(a1); c=fftshift(b); new=h.*c; new1=abs(ifft(new)); subplot(1,2,1); imshow(uint8(a1)); title('original Image'); subplot(1,2,2); imshow(uint8(new1)); title(['filtered image with radius=',string(d0)]);

Gaussian Low Pass Filter

Code:- a=imread('C:\Program Files\scilab-6.0.1\IPCV\images\cameraman.tif'); a1=double(a); r=size(a1,1); c=size(a1,2); d0=input('Enter the cut-off frequency - (Radius):-'); for u=1:1:r for v=1:1:c d=(((u-(r/2))^2)+((v-(c/2))^2))^0.5 h(u,v)=exp(-d*d/(2*d0*d0)) end end b=fft2(a1); c=fftshift(b); new=h.*c; new1=abs(ifft(new)); subplot(1,2,1); imshow(a);

title('Original Image'); subplot(1,2,2); imshow(uint8(new1)); title(['Filtered Image with radius = ',string(d0)]); new1=abs(ifft(new)); subplot(1,2,1); imshow(a); title('Original Image'); subplot(1,2,2); imshow(uint8(new1)); title(['Filtered Image with radius = ',string(d0)]);

Image Processing Practical #7

| Name | GORIMA SAYED | Roll Number | 21306A1054 |
| --- | --- | --- | --- |
| Subject/Cours e: | Image Processing |  |  |
| Topic | Image Sharpening in Frequency Domain |  |  |

Ideal High Pass Filter

Code:- /IdealhighPassFilter figure; p=imread("C:\Program Files\scilab-6.0.1\IPCV\images\cameraman.tif"); a1=double(p); r=size(a1,1); c=size(a1,2); d0=input('Enter the cut-off frequency-(Radius):-'); for u=1:1:r for v=1:1:c d=(((u-(r/2))^2)+((v-(c/2))^2))^0.5; if d<=d0 h(u,v)=0; else h(u,v)=1; end end end b=fft2(a1); c=fftshift(b); new=h.*c; new1=abs(ifft(new)); subplot(1,2,1);

imshow(uint8(p)); title('Original Image'); subplot(1,2,2); imshow(uint8(new1)); title(['Filtered Image with radius = ', string(d0)]); Output:-

Butterworth High Pass Filter

Code:- /Buterworthhighpassfilter

figure; a=imread("C:\Program Files\scilab-6.0.2\IPCV\images\cameraman.tif"); a1=double(a); r=size(a1,1); c=size(a1,2); d0=input('enter the cutt-of frequency-(Radius):-'); n=input("enter the order of the filter") foru=1:1:r forv=1:1:c d=(((u-(r/2))^2)+ ((v-(c/2))^2))^0.5; h(u,v)=1/(1+((d0/d)^(2^n))); end end b=fft2(a1); c=fftshift(b); new=h.*c; new1=abs(ifft(new)); subplot(1,2,1); imshow(uint8(a1)); title('original Image'); subplot(1,2,2); imshow(uint8(new1)); title(['filtered image with radius=',string(d0)]); Output:-

Gaussian High Pass Filter

Code:- figure; a=imread('C:\Program Files\scilab-6.0.2\IPCV\images\cameraman.tif'); a1=double(a); r=size(a1,1); c=size(a1,2); d0=input('Enter the cut-off frequency - (Radius):-'); for u=1:1:r for v=1:1:c d=(((u-(r/2))^2)+((v-(c/2))^2))^0.5 h(u,v)=1-exp(-d*d/(2*d0*d0)) end end b=fft2(a1); c=fftshift(b); new=h.*c; new1=abs(ifft(new)); subplot(1,2,1); imshow(a); title('Original Image'); subplot(1,2,2); imshow(uint8(new1)); title(['Filtered Image with radius = ',string(d0)]); Output:-

Image Processing Practical #8

| Name | GORIMA SAYED | Roll Number | 21306A1054 |
| --- | --- | --- | --- |
| Subject/Cours e: | Image Processing |  |  |
| Topic | Image Restoration |  |  |

Arithmetic Mean Filter to Remove Gaussian Noise

Code:- /arithmetic mean figure; a=imread('C:\Program Files\scilab-6.0.2\IPCV\images\blurry_moon.tif'); subplot(1,3,1) title("Original image") imshow(a) //Adding noise c=imnoise(a,'gaussian') / c=imnosie(a,g'aussain,'0.5)--Noisewithdenstiy subplot(1,3,2) title("Noisy image") imshow(c) d=double(c); b=d; [r1,c1]=size(a); for i=2;r1-1 for j=2:c1-1 a1=d(i-1,j-1)+d(i-1,j)+d(i-1,j+1)+d(i,j-1)+d(i,j)+d(i,j+1)+d(i+1,j-1)+d(i+1,j)+d(i+1,j+1); b(i,j)=a1*(1/9); //b(i,j)=a1*(1/9); end end subplot(1,3,3) title('Arithmetic filtered Image')

imshow(uint8(b)) Output:-

Geometric Mean Filter to Remove Gaussian Noise

Code:- /GeometricMean figure; a=imread("C:\Program Files\scilab-6.0.2\IPCV\images\blurry_moon.tif"); c=imnoise(a,'gaussian'); d=double(c); b=d; //backup [r1,c1]=size(a); for i=2:r1-1 for j=2:c1-1 a1=d(i-1,j-1)*d(i-1,j)*d(i-1,j+1)*d(i,j-1)*d(i,j)*d(i,j+1)*d(i+1,j)*d(i+1,j)*d(i+1,j+1); p=a1^(1/9); b(i,j)=p; end end subplot(1,3,1); imshow(a); title('Original Image');

subplot(1,3,2); imshow(c); title('noisy image'); subplot(1,3,3); imshow(uint8(b)); title('geometric mean filter'); Output:-

Harmonic Mean Filter to remove Gaussian / Salt and Pepper Noise

Code:- /harmonic meanwithGaussianNoise figure; a=imread('C:\Program Files\scilab-6.0.2\IPCV\images\blurry_moon.tif'); subplot(1,3,1) title("Original image") imshow(a) //Adding noise c=imnoise(a,'gaussian') / c=imnoise(a,g'aussian',0.5)-Noisewtihdensity subplot(1,3,2) title("Noisy image") imshow(c) d=double(c); b=d; [r1,c1]=size(a);

for i=2;r1-1 for j=2:c1-1 a1=(1/d(i-1,j-1))+(1/d(i-1,j))+(1/d(i-1,j+1))+(1/d(i,j-1))+(1/d(i,j))+(1/d(i,j+1))+(1/d(i+1,j- 1))+(1/d(i+1,j))+(1/d(i+1,j+1)); p=9/a1; b(i,j)=p; end end subplot(1,3,3) title('Filtered Image harmonic Mean Gaussian Noise') imshow(uint8(b)) Output:- /harmonic meanwithSaltandPepperNoise figure; a=imread('C:\Program Files\scilab-6.0.2\IPCV\images\blurry_moon.tif'); subplot(1,3,1) title("Original image") imshow(a) //Adding noise c=imnoise(a,'salt & pepper')/ c=imnoise(a',gaussain',0.5)-Noisewithdensity subplot(1,3,2) title("Noisy image") imshow(c) d=double(c); b=d; [r1,c1]=size(a);

for i=2;r1-1 for j=2:c1-1 a1=(1/d(i-1,j-1))+(1/d(i-1,j))+(1/d(i-1,j+1))+(1/d(i,j-1))+(1/d(i,j))+(1/d(i,j+1))+(1/d(i+1,j- 1))+(1/d(i+1,j))+(1/d(i+1,j+1)); p=9/a1; b(i,j)=p; end end subplot(1,3,3) title('Filtered Imageharmonic Meansaltand Pepper Noise') imshow(uint8(b)) Output:-

Contra Harmonic Mean Filter to Remove Salt & Pepper Noise

Code:- /contraharmonicmeantoremovesaltandpeppernoise figure; a=imread("C:\Program Files\scilab-6.0.2\IPCV\images\blurry_moon.png"); Q=input('Q:order of filter'); c=imnoise(a,'salt & pepper',0.002); d=double(c); Q=double(Q); [r1,c1]=size(a); for i=2:r1-1

for j=2:c1-1 N=(d(i-1,j-1)^(Q+1))+(d(i-1,j)^(Q+1))+(d(i-1,j+1)^(Q+1))+(d(i,j- 1)^(Q+1))+(d(i,j)^(Q+1))+(d(i,j+1)^(Q+1))+(d(i+1,j- 1)^(Q+1))+(d(i+1,j)^(Q+1))+(d(i+1,j+1)^(Q+1)); D=(d(i-1,j-1)^(Q))+(d(i-1,j)^(Q))+(d(i-1,j+1)^(Q))+(d(i,j- 1)^(Q))+(d(i,j)^(Q))+(d(i,j+1)^(Q))+(d(i+1,j-1)^(Q))+(d(i+1,j)^(Q))+(d(i+1,j+1)^(Q)); b(i,j)=N/D; end end subplot(1,3,1); imshow(a); title('Original Image'); subplot(1,3,2); imshow(c); title('Noisy Image'); subplot(1,3,3); imshow(uint8(b)); title('Contra Harmonic Mean Filtered Image'); Output:- Q=2 Q=-2

n (if Q is positive then black dots remove and if Q is negative then white dots remove)

Mid Point Filter to Remove Salt & Pepper Noise

Code:- figure; img=imread('C:\Program Files\scilab-6.0.1\IPCV\images\moon.tif'); c=imnoise(img,"gaussian",0.002) d=double(c); //b=d; //m=(1/9)*(ones(3,3)); [r1,c1]=size(c); for i=2:1:r1-1 for j=2:1:c1-1 a1=[d(i-1,j-1) d(i-1,j) d(i-1,j+1) d(i,j-1) d(i,j) d(i,j+1) d(i+1,j-1) d(i+1,j) d(i+1,j+1)];

amin=min(min(a1)); amax=max(max(a1)); newmin(i,j)=double((amin+amax)/2); end end subplot(2,2,1); imshow(img); title("Original Image"); subplot(2,2,2); imshow(c) title("After adding Salt and Pepper Noise in Original Image") subplot(2,2,3); imshow(uint8(newmin)); title("FilteredImageusing Min Filter") Output:-

Image Processing Practical #9

| Name | GORIMA SAYED | Roll Number | 21306A1054 |
| --- | --- | --- | --- |
| Subject/Cours e: | Image Processing |  |  |
| Topic | Color Image Processing |  |  |

Display different color planes and plot histogram of color image

Code:- clc; clear all; a=imread("C:\Program Files\scilab-6.0.2\IPCV\images\peppers.png"); R=a(:,:,1); G=a(:,:,2); B=a(:,:,3); subplot(2,2,1); imshow(a); title("Original Image"); subplot(2,2,2); imshow(R); title("Red Component Image"); subplot(2,2,3); imshow(G); title("Green ComponentImage"); subplot(2,2,4); imshow(B); title("Blue Component Image"); figure; //GethistValues for eachchannel [yR,x]= imhist(R);

[yG,x]= imhist(G); [yB,x]= imhist(B); //Plot them together in one plotx plot(x,yR,x,yG,x,yB,"LineWidth",2); xlabel("RGB Intensity"); ylabel("No of Pixels"); set(gca(),"grid",[1,1]) Output:-

Convert RGB image to HSV and CMY model

Code:- figure; a=imread("C:\Program Files\scilab-6.0.1\IPCV\images\peppers.png"); subplot(2,2,1); title("Original Image") imshow(a); HSV=rgb2hsv(a); subplot(2,2,2); title("RGB2HSV"); imshow(HSV); CMY=imcomplement(a); subplot(2,2,3); title("RGB2CMY") imshow(CMY); Output:-

Color Image Smoothing

Code:- //colorimagesmoothing clc; clear all; figure;

I=imread("C:\Program Files\scilab-6.0.1\IPCV\images\peppers.png"); F = fspecial('average',11); pic(:,:,1) = imfilter(I(:,:,1),F); pic(:,:,2) = imfilter(I(:,:,2),F); pic(:,:,3) = imfilter(I(:,:,3),F); Average_Image = uint8(pic); subplot(2,2,1); imshow(I); title('Original Image'); subplot(2,2,2); imshow(Average_Image); title('Average Filtered Image'); I=imnoise(I,'gaussian',0.07); F1 = fspecial('gaussian',3); pic1(:,:,1) =imfilter(I(:,:,1),F1); pic1(:,:,2) = imfilter(I(:,:,2),F1); pic1(:,:,3) = imfilter(I(:,:,3),F1); GLPF_Image = uint8(pic1); subplot(2,2,3);imshow(I);title('Noisy Image(Gaussian Noise)'); subplot(2,2,4);imshow(GLPF_Image);title('GLPF Filtered Image') Output:-

Color Image Sharpening

Code:- /colorimagesharpening clc; clear all; figure; I=imread("C:\Program Files\scilab-6.0.1\IPCV\images\peppers.png"); F=fspecial('prewitt'); pic(:,:,1) = imfilter(I(:,:,1),F); pic(:,:,2) = imfilter(I(:,:,2),F); pic(:,:,3) = imfilter(I(:,:,3),F); Sharp_Image = uint8(pic); subplot(1,2,1);imshow(I);title('Original Image'); subplot(1,2,2); imshow(Sharp_Image); title('Sharp Filtered Image') Output:-

Image Processing Practical #10

| Name | GORIMA SAYED | Roll Number | 21306A1054 |
| --- | --- | --- | --- |
| Subject/Cours e: | Image Processing |  |  |
| Topic | Morphological Image Processing |  |  |

Erosion

Code:- /Programtoapplyerosion clc; clear all; figure; a=imread('C:\Program Files\scilab-6.0.1\IPCV\images\morpex.png'); d=a; [r,c]=size(d); m=ones(3,3); for i=2:1:r-1 for j=2:1:c-1 new=[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A1(i,j)=min(new); end end subplot(1,2,1); imshow(a); title('Original Image'); subplot(1,2,2);

imshow(A1); title('Erosion Image'); Output:-

dilation

Code:- /Programtoapplydilation clc; clear all; figure; a=imread('C:\Program Files\scilab-6.0.1\IPCV\images\morpex.png'); d=a; [r,c]=size(d); m=ones(3,3); for i=2:1:r-1 for j=2:1:c-1 new=[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))];

A1(i,j)=max(new); end end subplot(1,2,1); imshow(a); title('Original Image'); subplot(1,2,2); imshow(A1); title('Dilation Image'); Output:-

Opening

Code:- figure; a=imread('C:\Program Files\scilab-6.0.1\IPCV\images\line.tif'); d=a; [r,c]=size(d); m=ones(3,3); fori=2:1:r-1 for j=2:1:c-1 new=[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j))

(m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A1(i,j)=min(new); end end d=A1; [r,c]=size(d); fori=2:1:r-1 for j=2:1:c-1 new=[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A2(i,j)=max(new); end end subplot(1,2,1); imshow(a); title('Original Image'); subplot(1,2,2); imshow(A2); title('processed image - o'); Output:-

Closing

Code:-

/ProgramtoapplyProcessedImage-Closing clc; clear all; figure; a=imread('C:\Program Files\scilab-6.0.1\IPCV\images\circbw.png'); d=a; [r,c]=size(d); m=ones(3,3); fori=2:1:r-1 for j=2:1:c-1 new=[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A1(i,j)=max(new); end end d=A1; [r,c]=size(d); fori=2:1:r-1 for j=2:1:c-1 new=[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A2(i,j)=min(new); end end subplot(1,2,1); imshow(a); title('Original Image'); subplot(1,2,2); imshow(A2); title('processed image - o'); Output:-

Boundary Extraction

Code:- Boundary clc; clear a= imread('C:\Program Files\scilab-6.0.1\IPCV\images\boundary.tif'); d=a; [r,c]=size(d); m= ones(3,3)

Vidyalankar School of Information Technology

fori=2:1:r-1 for j=2:1:c-1 new =[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))]; A1(i,j)=min(new); aa(i,j)= d(i,j) - A1(i,j) end end subplot(1,2,1) imshow(a); title("Original Image") subplot(1,2,2) imshow(aa) title("Processed Image Boundary Extraction")

Morphological Gradient

Code:- clc; Clear a= imread('C:\Program Files\scilab-6.0.1\IPCV\images\boundary.tif'); d=a; [r,c]=size(d); m= ones(3,3) for i=2:1:r-1 for j=2:1:c-1 new =[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))] A1(i,j)=max(new); End end d=A1; [r,c]= size(d) for i=2:1:r-1

for j=2:1:c-1 new =[(m(1)*d(i-1,j-1)) (m(2)*d(i-1,j)) (m(3)*d(i-1,j+1)) (m(4)*d(i,j-1)) (m(5)*d(i,j)) (m(6)*d(i,j+1)) (m(7)*d(i+1,j-1)) (m(8)*d(i+1,j)) (m(9)*d(i+1,j+1))] A2(i,j)=min(new); aa(i,j)= A1(i,j)-A2(i,j) end end subplot(1,2,1) imshow(a); title("Original Image") subplot(1,2,2) imshow(aa) title("Processed Image Gradient")

| Image Processing Practical #10 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Name | GORIMA SAYED | Roll Number | 21306A1054 |
|  | Subject/Cours e: | Image Processing |  |  |
|  | Topic | Image Segmentation |  |  |
|  | Edge Detection –Sobel operator |  |  |  |

# /Firstorderderivativefilter-SobelOperator

/prewit clc; clear all;

p=imread('C:\Program Files\scilab-6.0.1\IPCV\images\edge2.tif') figure;

subplot(2,2,1); imshow(p); title('Original Image');

;

v1=[1,0,-1;2, 0, -2;1, 0, -1]; /x-direction

h1=[-1, -2, -1; 0, 0 ,0; 1, 2, 1 ]; //y-direction

v2=[-1, 0, 1; -2, 0, 2; -1, 0, 1];

h2=[1, 2, 1; 0, 0, 0; -1, -2 ,-1];

gv1=abs(imfilter(double(p),v1));

# /subplot(3,2,2);

/imshow(gv1);

/title(V' ericalEdges)'; gh2=abs(imfilter(double(p),h2)); gh1=abs(imfilter(double(p),h1)); subplot(2,2,2);

imshow(gh1);

Vidyalankar School of Information Technology

title('horizontal Edges'); gv2=abs(imfilter(double(p),v2)); subplot(2,2,3);

imshow(gv2); title('vertical2 Edges');

finaledge=gh1+gv2+gh2+gv1; subplot(2,2,4); imshow(finaledge);

title('Final Image');

|  | Edge Detection –Canny Algorithm |
| --- | --- |
|  | clc; clear all; p=imread('C:\Program Files\scilab-6.0.1\IPCV\images\edge2.tif') figure; subplot(2,2,1); imshow(p); title('Original Image'); d=double(p); thresh=0.02; |

|  | sigma=3; / 3,5,7 E=edge(d, 'canny', thresh, sigma); subplot(2,2,2); imshow(E); title('Canny Edge Image'); /Firstorderderivativefilter-SobelOperator /prewit clc; clear all; p=imread('C:\Program Files\scilab-6.0.1\IPCV\images\edge2.tif'); figure; subplot(3,2,1); imshow(p); title('Original Image'); ; v1=[1,0,-1;2, 0, -2;1, 0, -1]; //x-direction h1=[-1, -2, -1; 0, 0 ,0; 1, 2, 1 ]; //y-direction |
| --- | --- |

v2=[-1, 0, 1; -2, 0, 2; -1, 0, 1];

h2=[1, 2, 1; 0, 0, 0; -1, -2 ,-1];

gv1=abs(imfilter(double(p),v1)); subplot(3,2,2);

imshow(gv1); title('Verical Edges');

gh2=abs(imfilter(double(p),h2)); gh1=abs(imfilter(double(p),h1)); subplot(3,2,3);

imshow(gh1); title('horizontal Edges');

gv2=abs(imfilter(double(p),v2)); subplot(3,2,4);

imshow(gv2); title('vertical2 Edges');

finaledge=gh1+gv2+gh2+gv1; subplot(3,2,5); imshow(finaledge);

title('Final Image');

Vidyalankar School of Information Technology

Vidyalankar School of Information Technology